LD2410_HCSR04_LED_test

Feeding the Uno from the same supply ties all grounds together automatically. If you power the Uno from USB instead, run a wire from the supply’s − terminal to any Uno GND pin.

### Pin map

| Device pin | Connects to |
| --- | --- |
| HC-SR04 VCC | Uno 5V |
| HC-SR04 Trig | Uno D7 |
| HC-SR04 Echo | Uno D8 |
| HC-SR04 GND | Uno GND |
| LD2410 5V | Uno 5V |
| LD2410 OUT | Uno D2 |
| LD2410 GND | Uno GND |
| LD2410 TX / RX | Leave unconnected |
| Strip A DIN | 330 Ω resistor, then Uno D5 |
| Strip B DIN | 330 Ω resistor, then Uno D6 |
| Strip A and B +12V | Supply + (through the fuse) |
| Strip A and B GND | Supply − |

### Build order

- Upload the sketch over USB with the strips still disconnected. Open the Serial Monitor at 115200 and uncomment the Serial.println(cm) line to check the ultrasonic readings.

- Connect the strips. Wire +12V and GND first, then the data lines through the 330 Ω resistors.

- Power up and test. Walk toward the sensors. Cyan on strip A means the ultrasonic triggered, and magenta on strip B means the radar triggered.

### Things to check

- Data direction. Follow the arrows printed on each strip. Data enters at the DIN end.

- Wrong colors. Change COLOR_ORDER in the sketch to BRG or GRB.

- Nothing lights. Check the common ground first, then the data arrow direction.

- LD2410 placement. Mount it facing the approach path, not behind metal. It also detects stationary people, so strip B will stay lit longer than strip A. Set its range with HLK’s phone app.

- Barrel jack heat. The Uno’s regulator may run warm at 12V. The load here is light, but a small 12V→5V buck converter feeding the 5V pin is the cooler alternative.