Ceiling 1 Test: Nano, MAX9814 triggers angry 5V LED’s and servo

- 1

Power the LEDs and servo from an external 5V supply20 WS2812 pixels can draw up to about 1.2A at full white (roughly 60mA each), and the servo adds current spikes when it moves. Use a separate 5V supply rated for at least 3A and feed the LED strip's 5V and GND directly from it. Don't power the strip from the Nano's 5V pin or from USB.

- 2

Tie all grounds togetherConnect the external supply's GND, the Nano's GND, the LED strip's GND, the servo's ground wire, and the MAX9814's GND together. Without a shared ground, the data and audio signals have no common reference and you'll get flicker or random triggering.

- 3

Wire the LED strip data lineRun a wire from Nano pin 5 to the strip's data-in (DIN) pin through a 300-500 ohm resistor placed close to the strip. Add a 1000 uF capacitor (6.3V or higher) across the strip's 5V and GND at its input, watching polarity on the capacitor.

- 4

Wire the servoServo signal (usually orange or yellow) goes to Nano pin 9. Servo ground (brown or black) goes to the common ground. Servo power (red) goes to the external 5V supply, not the Nano's 5V pin, since servos draw sudden current spikes that can brown out the Nano.

- 5

Wire the MAX9814 microphoneMAX9814 VDD goes to the Nano's 5V pin (it draws only a few milliamps), GND goes to common ground, and OUT goes to Nano pin A0. For the GAIN pin, tie it to VDD for the lowest gain (40dB), leave it floating for 60dB, or tie it to GND for 50dB. Start with GAIN tied to VDD (40dB) to keep room noise low, and raise gain only if the mic seems deaf. Leave AR and SHDN unconnected for default behavior (SHDN can be tied to VDD if your breakout needs it to stay enabled).

- 6

Route wires to keep audio cleanKeep the microphone's OUT wire short and away from the LED data wire and servo wires, since the LED data line and servo pulses can couple noise into the audio signal and cause false triggers. Mounting the mic away from the servo also reduces vibration pickup.

- 7

Leave A1, pin 0, and pin 1 aloneLeave A1 unconnected (it seeds the random generator) and leave pins 0 and 1 free for the DFPlayer Mini later. Power the Nano from USB while developing, then from VIN (7-12V) or the shared 5V rail in the final build, keeping grounds common either way.